COMMENTARY BY INTER COUNTY SURVEYS ON THE BUILDING WORKS CARRIED OUT AT 21 – 23 CHURCH STREET,

BASINGSTOKE RG21 7QQ

Client: 			Mr Ahmet Topcu

21 – 23 Church Street

Basingstoke

RG21 7QQ

Proposed Work: 	Erection of 2 no. first floor extensions and 2 no. external

staircases, to create 3 no. 1 bed and 2 no. 2 bed

dwellings, and a restaurant with outdoor seating area| 21

and 23 Church Street Basingstoke RG21 7QQ

Location: 		21 – 23 Church Street

Basingstoke

RG21 7QQ

Agent: 			Inter County Surveys

4 Suffolk Road

Andover

Hants

SP10 2JD

Ref: 			21965

Date: 			2nd May 2023

© Inter County Surveys

Section 1

Introduction

1.10 	This document has been prepared to assist in the understanding of the building

works carried out during an extensive 3½ year construction period, to turn the

existing empty derelict and dilapidated, water- and rot-damaged building into the

successful and popular Turkish and Mediterranean restaurant.

1.11 	The premisses are run by husband and wife, Ahmet and Marta Topcu, with their

very dedicated team. They are passionate about creating wonderful

food and memorable experiences. From hand-made breads to the extensive

wine list, they have taken great care and effort on every aspect of the menu.

They are passionate, local and independent family run business.

1.12 	The Olive House is now listed as number 1 out of 147 ‘Top Restaurants in

Basingstoke’ on the Trip Advisor website as of 7th May 2021.

1.13 	The journey began back in 2016, when the premisses were bought by Mr & Mrs Topcu, with the vision of opening a special and quality Turkish and Mediterranean restaurant. We must say at this point that as one of the people first through the door when Ahmet took procession, the vision was a little hard for us to imagine.

1.14	The building is a Grade II listed building.

1.15 	The original proposals involved a change of use from a former bar/nightclub to

restaurant and part-residential use with associated internal and external

alterations, and two 1st floor rear extensions to no.21 and no.23. The ground

floor retail use in the shop at no.23 will continue. The new residential use would

comprise one 1-bedroom flat and one 2- bedroom flat to the 1st floor of no.21

and one 1-bedroom staff flat to the 2nd floor of no.21 and one studio ground

floor flat to the rear of no.23 and one 2-bedroom flat to the 1st floor no.23. The

proposals also included an external rear staircase to no.21 to provide access to the

1st floor flat and a further external staircase to the 1st floor of the brewhouse

flat. The four flats will be for rent rather than sale and the staff flat will be

maintained in that use as there is only direct access from the restaurant.

1.16 	These proposals can be seen from the original approved drawings contained

within application 16/01586/LBC, and 16/01585/FUL. There were also series of

full and listed building applications during the course of the works, as the true

extent of the years of decay and neglect, and structurally compromised

elements of the building, were uncovered as the works proceeded.

1.17	Drawing ICS/1968/58 has been prepared to show the layout of the building

following the completion of the works, if the works in this sort and age of

building can ever be considered as complete.

Section 2

Ground Floor

Front restaurant

2.10 	The internal layout of the front part of the restaurant is basically as shown on

the original drawings, with only the furniture layout altering. The reception area,

is just that, but drinks, and coffees are prepared from here as well. The ‘Bar’

area has been replaced with cover tables. The food orders bar has been omitted.

2.11 	The private door to the flat above the restaurant has been blocked up as this

would not conform to current fire regulations and means of escape, further

detail on this is set out in section 2.24.

2.12 	The biggest change on the ground floor is where the kitchen and toilets have

been repositioned. The proposed kitchen and food preparation area have been

relocated into the end of the building where the toilets and storage and service

area is shown. There were a number of reasons for this, 1) it has provided a much

larger kitchen floor area to service the number of covers being provided, 2) from

an environmental point of view it was considered not to have the public passing a

food preparation area to get to the toilets, 3) the revised layout provided an

increased number and better layout for the tables, and 4) the number of toilets

originally shown was not required.

2.13	The original ceiling lining (plasterboard) to the new kitchen area was removed

as a result of water damage and the result removing the old toilets and

blockwork walls. A number of morticed Oak timber beams were discovered

providing support to the other softwood joists. Following discussions with the

Local Authority (see listed building application), the beams in question had

again suffered extensive damage due to the ingress of water. Because the beam ends had been built into solid walls, their ends had suffered catastrophic rot.

It was agreed and approved that these beams be replaced with matching details

and mouldings as the originals, in oak. The existing softwood joists that formed

the ceiling of this area were removed, cleaned inspected, and were re-used for

the new floor of Flat 4 (dining/living and kitchen). New softwood joists were

then installed to form the support for the new ceiling in the new kitchen. The

new Oak beams have once again been enclosed within the floor ceiling void.

2.14 	At the front of the rear restaurant area, additional support posts have been

installed to provide structural support to the (existing) rear walling at first floor

level, and ultimately the roof structure above. These posts are provided with

isolated pad foundations and have been located adjacent existing timber posts

forming part of the original timber frame structure. The new posts have been

clad in reclaimed timber and finished to match the existing.

2.15 	In the floor area adjacent to the door leading to the staff flat above, a glass

‘viewing’ panel has been installed into the floor. This provides an insight into

part of the original cellar area. An opening has been made through the existing

modern softwood joists to install the glass panel. Subtle lighting introduced into

this part of the cellar area. This has proved to be a very popular feature of the

restaurant, with tables adjacent are at a premium.

Area to the rear of the Shop (Flat 1)

2.16 	Changes have been made to this area, this area is now a ‘private’ restaurant area,

providing a more secluded area for private functions. The whole area within this

part of the building was found to be in a very serious state of dis-repair, with

many of the existing features previously having been covered up with modern

timber studwork and plasterboard. These have now been exposed and restored.

The wall between what would have been the living room and the kitchen

dining area (studwork walling) has been removed leaving the original timber

framing support posts and beams in place. Within this area all the existing

ceiling beams, and an original metal support post have been restored back to

their original state, including a carved beam over the reception area and pot

wash. The wash-room has also had a complete ‘make over’.

2.17 	The rear external door and window to the area above has been replaced, as the

original door/frame and window were not serviceable.

External

2.18 	There are a few changes to the outside of the building, some of which we will

detail later in this document.

A number of the windows that view the inner courtyard from the rear restaurant

have been replaced. The original windows were of a modern softwood stormproof

sash type windows double glazed, which were completely rotten. These have been replaced by similar softwood side-hung casement windows.

2.19	The original approved external 2No staircases shown leading to the first-floor flats

have been re-configured into one access stairway, providing the required escape

routes in accordance with the current building regulations. Planning and listed

building consent applications were submitted for these alterations.

2.20 	A link way has been provided between the rear doors at the back of the

restaurant and the private function restaurant. This link provides the required

fire escape requirements of both the ground floor restaurant and the first floor

flat at the front of the building. The link enables food and required food

hygiene requirements, without having to go outside the premisses.

2.21 	The existing ‘garage’ structure to the Northwest of the site, provides a W/C for

use by the staff members, and an area for refrigerated and frozen food products.

The existing above ground drainage has been modified to suit the new layout.

First Floor

Flat 2 (as originally)

2.22	The changes made in this particular flat is that the new door opening proposed

To gain access into Bedroom 1 has not been done. This bedroom 1 is now a staff restroom, to which we refer in item 2.26.

2.23 	The main changes within Flat 2 are that the kitchen has been re-positioned

within the entrance end of the main living room to form a kitchen/dining living

room. The old kitchen location is bedroom 2, and bedroom 2 is retained.

The original fireplace that was going to be opened up, has not been for safety

reasons. The existing internal door opening that would have led into Flat 3, has

been infilled. The three very large sliding sash windows to the front elevation,

the sashes have been removed, stripped and repaired and re-decorated. Sash

cords replaced and generally overhauled.

Flat 3 (as originally), (note the flat on the second floor now forms part of flat 3)

2.24 	The opening between the kitchen/diner and the stairs leading up to the second

floor and down to the ground floor has not been blocked up, but has been fitted

with a fire resisting door. An additional partition has been fitted across the

corner of the kitchen/dinner and fitted with a fire door to provide the requirements

of building control and the fire officer.

The opening in the floor formed by the stairs leading down to the ground floor

has been filled in with floor joists and flooring.

The opening originally shown that opened into a lobby between the doors

leading into the living room and the bedroom has not been fitted. These other

two doors are fire doors.

2.25 	The bathroom has been fitted out as proposed, however when removing some of

the existing plasterboard wall lining, an existing fireplace and surround was

discovered. This has now been left exposed, after being cleaned and refurbished.

The brickwork to this chimney breast has been cleaned, sealed and again left

exposed.

Staff Restroom

2.26 	As we have mentioned in 2.22 above, this area of the building has been fitted

out as a staff restroom, due to the fact that the second floor flat could not be used for

this purpose. The internal arrangement is a bit different to that approved,

together with the relocation of the window in the front end, and the inclusion of

the entrance door in the same gable end.

Access to this self-contained flat is via the re-configured rear access and escape

staircase way. This restroom is contained within the first floor extension approved

under the original planning and listed building consent.

Flat 4 (as originally)

2.27	 The original roof on this structural originally housed all the mechanical plant

and large mechanical extract ducting for the original kitchen. The roof has

suffered extensive water ingress, and the damage and rot that had taken place,

thus ingress must have been happening for many, many, years. With the

damage that had been caused, the entire roof structure had to be removed.

2.28	The internal courtyard side of the first-floor external walling, was a patchwork

of timber studwork and external timber cladding. This also had significate water

damage, and excessive structural movement as a result of this damage. None of

this studwork walling has any buttressing support. It was commented by the

structural engineers that the whole structure was one snow-storm away from

total collapse.

The rear (rear wall on the common boundary), front facing (car park) and back

facing wall of the original roof structural, none of these walls were tied to each

other, each of them effectively free standing and unsafe. These walls in part had

to be rebuilt, tying them in at the corners, and providing steel strapping and ties

where they could not be rebuilt.

The wall between the dining/living and Bedroom 1 had to be rebuilt in its

entirety.

2.29	The internal layout of this flat is largely as proposed, but, with the addition

of a timber studwork partition to separate the kitchen, dining/living room thus to

provide the means of escape from the adjacent bedrooms. Again, this was a

requirement of the fire officer.

The window in the front gable end (facing the carpark), it has been reported by

the local authority that this was a new window opening. It was not. There was a

slightly smaller window that had been filled-in many years ago. This infill has

been removed, the window opening made slightly wider as shown on the

approved drawings, and a new window installed.

Second Floor

Flat 4 (as originally)

2.30 	The basic layout has not changed from the original proposals, but the two

proposed rear facing dormer windows have been with conservation roof

windows. Installing dormer windows would have required a significate amount

of the original historic framing to have been removed.

2.31 	The two front-facing small dormer windows in the living room, and bedroom

have been replaced as proposed with details to match the existing.

2.32 	Several the existing ceilings being lath and plaster, had shown characteristics of

de-lamination from the framing above. To avoid the removal of historical

elements, these ceilings have been ‘over-boarded’ with plasterboard.

End